Andrew Wells

CPE301 – SPRING 2016

# Design Assignment 1

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

| 1. | INITIAL CODE OF TASK 1/A |  |  |
| --- | --- | --- | --- |

; Andrew Wells CPE 301 DA1

; DA1T1.asm

;

; Created: 2/24/2016 9:26:24 PM

; Author: sirfe

;

; Macro to create the stack

.macro SET_STACK	;Beginning of macro

ldi	r16, LOW(RAMEND); Copy the lower 8 bits of the end of the RAM into R16

out	spl, r16; Copy the address in r16 to the lower portion of the stack pointer

ldi	r16, HIGH(RAMEND); Copy the higher 8 bits of the end of the RAM into R16

out	sph, r16; Copy the address in r16 to the higher portion of the stack pointer

.endmacro

.cseg

.org 0

rjmp start; Skips over the interrupt portion of the memory

.org 0x20	; starting point for the program

start:

SET_STACK	; runs macro to create the stack

ldi	r19, 25; Copy 25 into R19, keeps track of how many values have been obtained

ldi	r20, 0	; make sure R20 is 0 to start

ldi	r21, 0	; make sure R21 is 0 to start

ldi	r23, 0	; make sure R23 is 0 to start

ldi	r24, 0	; make sure R24 is 0 to start

ldi	ZH, HIGH(RAMEND); Copy the High portion of the end of the RAM to ZH pointer/register

ldi	ZL, LOW(RAMEND); Copy the Lower portion of the end of the RAM to the ZL pointer/register

; Divide by 2, to get to the middle of the RAM

lsr	ZH; Shift the upper portion of Z to the right

ror	ZL; Rotate the lower portion of Z to the right, brings in the carry(if any) from the upper portion

NEXT:

mov	r17, ZL; Copy the value of the lower portion of the address in Z to R17

st	Z+, r17; Store back the value into the RAM and increment Z to the next location

mov	r16, r17; Copy the value in R17 to R16, to keep the value available for later calculations

DIV7:

subi	r16, 7	; Subtract the Value in R16 by 7

cpi	r16, 7	; Check to see if the value in r16 is 7

brsh	DIV7	; If the value is greater than or equal to 7, jump to DIV7 and continue to subtract

cp	r16, r0; Compare the value in R16 to 0

brne	NEXTDIV; If R16 is not 0 Skip to NEXTDIV and do not add value to total

add	r20, r17; If value is divisible by 7 then add value to running total

adc	r21, r21; Add R21 to itself, only increases if there is a carry from previous addition

NEXTDIV:

mov	r16, r17; Copy the Value in R17 into R16

DIV3:

subi	r16, 3	; Subtract the Value in R16 by 3

cpi	r16, 3	; Check to see if the value in r16 is 3

brsh	DIV3	; If the value is greater than or equal to 3, jump to DIV3 and continue to subtract

cp	r16, r0; Compare the value in R16 to 0

brne	DIVDONE; If R16 is not 0 Skip to DIVDONE and do not add value to total

add	r23, r17; If value is divisible by 3 then add value to running total

adc	r24, r24; Add R24 to itself, only increases if there is a carry from previous addition

DIVDONE:

dec	r19	; Decrement R19 by 1

cpi	r19, 0	; Compare R19 with 0

brne	NEXT	; If not zero return to beginning and get another value

cpi	r21,0	; Compare R21 with 0

brne	SETREG	; If R21 is not 0 jump to SETREG

cpi	r24,0	; Compare R24 with 0

brne	SETREG	; If R24 is not 0 jump to SETREG

HOLD:

rjmp HOLD	; Infinite Loop

SETREG:

ldi	r16, 4	; Copy 4 into R16

mov	r7, r16; Copy 4 into R7, setting bit 3 to high

rjmp	HOLD	; Jump to infinite loop

| 2. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1/A:

Store 25 numbers starting from the RAMEND location. Capture the lower 8bits of the RAM MIDDLE = (RAMBEGIN + RAMEND)/2 address and use them as your values. You can increment or decrement rom RAM MIDDLE location to get the subsequent 24 numbers. Use the X/Y/Z registers to fill up the stack of 25 numbers.

Before:

After:

TASK1/B:

Parse through the 25 numbers and add all numbers divisible by 7 and place the

result in R20:21.

TASK1/C:

Parse through the 25 numbers and add all numbers divisible by 3 and place the

result in R23:24. Parsing of the numbers for task b and c has to be done

simultaneously.

Before:

After:

TASK1/D:

Check and set register R07.3 if the sum is greater than 16-bits.

Before:

After:

TASK1/E:

Determine the execution time @ 16MHz/#cycles of your algorithm using the

simulation.

| 3. | Flowchart |  |  |
| --- | --- | --- | --- |

True

R16 >= 7

R16 = R16 - 7

Move value from R17 into R16

Load value from X into R17 Increment X

True

False

Is R19 = 0

Decrement R19

Store Value into Z memory location, increment Z

Copy lower portion of Z address into R17

Load RAMEND into Z Copy Z into X to save

Load 25 into R19

Create Stack

START

False

R16 = 0

False

True

R20 = R20 + R17

R21 = R21 + Carry

Move value from R17 into R16

R16 = R16 - 3

True

R16 >= 3

False

False

R16 = 0

True

R23 = R23 + R17

R24 = R24 + Carry

Decrement R19

False

R19 = 0

True

R21 != 0

False

R24 != 0

False

HOLD

Load 4 into R16

Move R16 into R7

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| No Video Taken |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| https://github.com/Wellsa15/wellsa_unlv |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Wells