Andrew Wells

CPE301 – SPRING 2016

# Design Assignment 2

DO NOT REMOVE THIS PAGE DURING SUBMISSION:

The student understands that all required components should be submitted in complete for grading of this assignment.

| NO | SUBMISSION ITEM | COMPLETED (Y/N) | MARKS (/MAX) |
| --- | --- | --- | --- |
| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| 1. | INITIAL CODE OF TASK 1/A |  |  |
| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2/B |  |  |
| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3/C |  |  |
| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4/D |  |  |
| 5. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 5/E |  |  |
| 6. | SCHEMATICS |  |  |
| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| 8. | SCREENSHOT OF EACH DEMO |  |  |
| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |

| 0. | COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS |  |  |
| --- | --- | --- | --- |

Xplained Mini: ATmega328P micro controller

10 Segment LED bar graph

10 100Ω Resistors

| 1. | INITIAL CODE OF TASK 1 |  |  |
| --- | --- | --- | --- |

.macro MAKESTACK				;Start of Stack Macro

ldi		r16, LOW(RAMEND)	;Copy the lower 8 bits of the end of the RAM into R16

out		spl, r16			;Copy the address in r16 to the lower portion of the stack pointer

ldi		r16, HIGH(RAMEND)	;Copy the higher 8 bits of the end of hte RAM into R16

out		sph, r16			;Copy the address in r16 to the higher portion of the stack pointer

.endmacro

.cseg

.org 0x0

jmp		START

.org 0x100		;Start of Program

START:

SBI		DDRC, 0	;Enable DDRC.0

SBI		DDRC, 4	;Enable DDRC.4

SBI		DDRC, 5	;Enable DDRC.5

LDI		R21, 0	;Set R21 to 0

LDI		R22, 0	;Set R22 to 0

LDI		R16, 0xFF	;Load 0xFF into R16

OUT		DDRB, R16	;Enable all pins of DDRB

MAKESTACK	; Call MAKESTACK macro to create the stack

COUNTING:

call	DELAY	;Run DELAY function

CPI		R17, 0x01	;Compare R17 with 1

BRNE	COUNTING	;If R17 is 0 go back to COUNTING and run DELAY again

INC		R21		;Increase R21 by 1

INC		R22		;Increase R22 by 2

OUT		PORTB, R21	;Copy value of R21 into PORTB

CHECK1:

CPI		R22, 5	;Compare R22 with 5

BRNE	CHECK2	;If not 5 skip to CHECK2

LDI		R18, 0x20;Set R18 to 0x20, 0010 0000

OR		R18, R17; OR R18 and R17, Keeps value in PORTC.0

OUT		PORTC, R18;Copy R18 into PORTC

rjmp	COUNTING ; Return to COUNTING to repeat loop

CHECK2:

CPI		R22, 10	;Compare R22 with 10

BRNE	CHECK3	;If not 10 skip to CHECK3

LDI		R18, 0x10;Set R18 to 0x10, 0001 0000

OR		R18, R17; OR R18 and R17, Keeps Value in PORTC.0

OUT		PORTC, R18; Copy R18 into PORTC

rjmp	COUNTING ;Return to COUNTING to repeat loop

CHECK3:

CPI		R22, 15	;Compare R22 with 15

BRNE	CHECK4	;If not 15 skip to CHECK4

LDI		R18, 0x30; Set R18 to 0x30, 0011 0000

OR		R18, R17; OR R18 and R17, Keeps value in PORTC.0

OUT		PORTC, R18;Copy R18 into PORTC

rjmp	COUNTING;Return to COUNTING to repeat loop

CHECK4:

CPI		R22, 20	;Compare R2 wtih 20

BRNE	COUNTING; If not 20 Return to COUNTING to repeat loop

LDI		R22, 0	; Reset R22 to 0

LDI		R18, 0x00;Load 0 into R18

OR		R18, R17; OR R18 and R17, Keeps Value in PORTC.0

OUT		PORTC, R18; Copy R18 into PORTC

rjmp	COUNTING; Return to COUNTING to repeat loop

HOLD:

rjmp	HOLD			;Infinite Loop

DELAY:

LDI		R16, 0x01	;Set R16 to 1

LDI		R20, 0xF0	;When using Chip on breadboard: TCNT1 = 0xF862, 63586

STS		TCNT1H, R20	;When using Xplained Mini board: TCNT1 = 0xF0BE, 61630

LDI		R20, 0xBE	;When using Chip on breadboard: TCNT1H = 248, TCNT1L = 98

STS		TCNT1L, R20	;When using Xplained Mini board: TCNT1H = 240, TCNT1L = 190

LDI		R20, 0x00	;Set R20 to 0, 00000000

STS		TCCR1A, R20	;Normal Mode

LDI		R20, 0x05	;Set R20 to 5, 00000101

STS		TCCR1B, R20 ;1024 prescaler

LOOP1:

IN		R20, TIFR1	;Copy TIFR1 into R20

SBRS	R20, TOV1	;skip next instruction if TOV1 = 1

rjmp	LOOP1		;Jump back to LOOP1

EOR		R17, R16	;XOR R17 and R16 and Save into R17

CPI		R17, 0x00	;Compare R17 with 0

BRNE	NOTZERO		;Jump to NOTZERO if R17 is not zero

CBI		PORTC, 0	;Set PORTC.0 to 0

rjmp	ZERO		;Jump to ZERO

NOTZERO:

SBI		PORTC, 0	;Set PORTC.0 to 1

ZERO:

LDI		R19, 1<<TOV1;Clear TOV1

OUT		TIFR1, R19	;clear OCF1A

ret; Return to Main Program

| 2. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 2 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL // XTAL= 8MHZ

#include <avr/io.h>

void DELAY();// Delay function

unsigned char a = 1;// Set a to 1 for XOR with PORTC.0

unsigned char num = 0;// Set num to 0, counting Variable

unsigned char byfive = 0;// Set byfive to 0, counting Variable

int main(void)

{

DDRB = 0xFF;// Activate all PORTB Pins

DDRC = 0x31;// Activate PORTC Pins 0, 4, 5

while (1) //Infinite loop

{

DELAY();// Run Delay function

if ((PORTC % 2) == 1) //

{

num++;// Increase num by 1

byfive++;// Increase byfive by 1

PORTB = num;// Copy the value in num into PORTB

if (byfive == 5)// Compare byfive with 5

{

PORTC |= 1 << PORTC4;// Set PORTC.4 to 1

PORTC &= 0 << PORTC5;// Set PORTC.5 to 0

}else

if (byfive == 10)// Compare byfive with 10

{

PORTC &= 0 << PORTC4;// Set PORTC.4 to 0

PORTC |= 1 << PORTC5;// Set PORTC.5 to 1

}else

if (byfive == 15)// Compare byfive with 15

{

PORTC |= 1 << PORTC4;// Set PORT.4 to 1

PORTC |= 1 << PORTC5;// Set PORT.5 to 1

}else

if (byfive == 20)// Compare byfive with 20

{

byfive = 0;// Reset byfive to 0

PORTC &= 0 << PORTC4;// Set PORT.4 to 0

PORTC &= 0 << PORTC5;// Set PORT.5 to 0

}

}

}

return 0;

}

void DELAY()//  Function to cause delay

{

TCNT1 = 0xF0BE;// When using chip on breadboard: TCNT1 to 0xF862, 63586

// When using Xplained Mini board: TCNT1 = 0xF0BE, 61630

TCCR1A = 0x0;//Normal Mode

TCCR1B = 0x05;//1024 Prescaler

while ((TIFR1 & (1<<TOV1)) == 0)// Stay in loop while TOV1 = 0

{

}

TIFR1 |= 1 << 0;// Clear TOV1

PORTC ^= a << PORTC0;// XOR PORTC.0 with a to toggle on and off

return;	// Return to main program

}

| 3. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 3 |  |  |
| --- | --- | --- | --- |

.macro MAKESTACK				;Start of Stack Macro

ldi		r16, LOW(RAMEND)	;Copy the lower 8 bits of the end of the RAM into R16

out		spl, r16			;Copy the address in r16 to the lower portion of the stack pointer

ldi		r16, HIGH(RAMEND)	;Copy the higher 8 bits of the end of hte RAM into R16

out		sph, r16			;Copy the address in r16 to the higher portion of the stack pointer

.endmacro

.cseg

.org 0x0

jmp		START	;Jump to program start

.org 0x16

jmp		TIM1_COMPA	;Jump to Interrupt

.org 0x100		;Start of Program

START:

SBI		DDRC, 0	;Enable DDRC.0

SBI		DDRC, 4	;Enable DDRC.4

SBI		DDRC, 5	;Enable DDRC.5

LDI		R21, 0	;Set R21 to 0

LDI		R22, 0	;Set R22 to 0

LDI		R17, 0	;Set R17 to 0

LDI		R16, 0xFF	;Load 0xFF into R16

OUT		DDRB, R16	;Enable all pins of DDRB

MAKESTACK	; Call MAKESTACK macro to create the stack

LDI		R20, 0x02	;Set R20 to 2, 0000 0010

STS		TIMSK1, R20	;Enable Timer1 compare interrupt

SEI		;Global Interrupt Enable

LDI		R20, 0x0F	;When using Chip on breadboard: TCNT1 = 0x079E, 1950

STS		OCR1AH, R20	;When using Xplained Mini board: OCR1A = 0x0F42, 3906

LDI		R20, 0x42	;When using Chip on breadboard: TCNT1H = 7, TCNT1L = 158

STS		OCR1AL, R20	;When using Xplained Mini board: TCNT1H = 15, TCNT1L = 66

LDI		R20, 0x00	;Set R20 to 0, 00000000

STS		TCCR1A, R20	;CTC Mode

LDI		R20, 0x0D	;Set R20 to 13, 00001101

STS		TCCR1B, R20 ;1024 prescaler

HOLD:

rjmp	HOLD			;Infinite Loop

TIM1_COMPA: ;OCF1A clears automatically when interrupt is run

LDI		R16, 0x01	;Set R16 to 1

EOR		R17, R16	;XOR R17 and R16 and Save into R17

CPI		R17, 0x00	;Compare R17 with 0

BRNE	NOTZERO		;Jump to NOTZERO if R17 is not zero

CBI		PORTC, 0	;Set PORTC.0 to 0

rjmp	COUNTING	;Jump to COUNTING

NOTZERO:

SBI		PORTC, 0	;Set PORTC.0 to 1

INC		R21		;Increase R21 by 1

INC		R22		;Increase R22 by 2

OUT		PORTB, R21	;Copy value of R21 into PORTB

CHECK1:

CPI		R22, 5	;Compare R22 with 5

BRNE	CHECK2	;If not 5 skip to CHECK2

LDI		R18, 0x20;Set R18 to 0x20, 0010 0000

OR		R18, R17; OR R18 and R17, Keeps value in PORTC.0

OUT		PORTC, R18;Copy R18 into PORTC

jmp		COUNTING	;Jump to COUNTING, skip rest of checks

CHECK2:

CPI		R22, 10	;Compare R22 with 10

BRNE	CHECK3	;If not 10 skip to CHECK3

LDI		R18, 0x10;Set R18 to 0x10, 0001 0000

OR		R18, R17; OR R18 and R17, Keeps Value in PORTC.0

OUT		PORTC, R18; Copy R18 into PORTC

jmp		COUNTING	;Jump to COUNTING, skip rest of checks

CHECK3:

CPI		R22, 15	;Compare R22 with 15

BRNE	CHECK4	;If not 15 skip to CHECK4

LDI		R18, 0x30; Set R18 to 0x30, 0011 0000

OR		R18, R17; OR R18 and R17, Keeps value in PORTC.0

OUT		PORTC, R18;Copy R18 into PORTC

jmp		COUNTING	;Jump to COUNTING, skip rest of checks

CHECK4:

CPI		R22, 20	;Compare R2 wtih 20

BRNE	COUNTING; If not 20 Skip to COUNTING to exit interrupt

LDI		R22, 0	; Reset R22 to 0

LDI		R18, 0x00;Load 0 into R18

OR		R18, R17; OR R18 and R17, Keeps Value in PORTC.0

OUT		PORTC, R18; Copy R18 into PORTC

COUNTING:

RETI ; return to main program

| 4. | INCREMENTAL / DIFFERENTIAL CODE OF TASK 4 |  |  |
| --- | --- | --- | --- |

#define F_CPU 8000000UL // XTAL= 8MHZ

#include <avr/io.h>

#include <avr/interrupt.h>

unsigned char a = 1;// Set a to 1 for XOR with PORTC.0

unsigned char num = 0;// Set num to 0, counting Variable

unsigned char byfive = 0;// Set byfive to 0, counting Variable

int main(void)

{

DDRB = 0xFF;// Activate all PORTB Pins

DDRC = 0x31;// Activate PORTC Pins 0, 4, 5

OCR1A = 0x0F42;// When using Chip on breadboard: TCNT1 = 0x079E, 1950

//When using Xplained Mini board: OCR1A = 0x0F42, 3906

TCCR1A = 0x0;//CTC Mode

TCCR1B = 0x0D;//1024 Prescaler

TIMSK1 = (1<<OCIE1A); //Enable Timer1 compare interrupt

sei (); // Enable global interrupt

while (1) //Infinite loop

{

}

return 0;

}

ISR (TIMER1_COMPA_vect)

{

PORTC ^= a << PORTC0;// XOR PORTC.0 with a to toggle on and off

if ((PORTC % 2) == 1) //

{

num++;// Increase num by 1

byfive++;// Increase byfive by 1

PORTB = num;// Copy the value in num into PORTB

if (byfive == 5)// Compare byfive with 5

{

PORTC |= 1 << PORTC4;// Set PORTC.4 to 1

PORTC &= 0 << PORTC5;// Set PORTC.5 to 0

}else

if (byfive == 10)// Compare byfive with 10

{

PORTC &= 0 << PORTC4;// Set PORTC.4 to 0

PORTC |= 1 << PORTC5;// Set PORTC.5 to 1

}else

if (byfive == 15)// Compare byfive with 15

{

PORTC |= 1 << PORTC4;// Set PORT.4 to 1

PORTC |= 1 << PORTC5;// Set PORT.5 to 1

}else

if (byfive == 20)// Compare byfive with 20

{

byfive = 0;// Reset byfive to 0

PORTC &= 0 << PORTC4;// Set PORT.4 to 0

PORTC &= 0 << PORTC5;// Set PORT.5 to 0

}

}

}

| 6. | SCHEMATICS |  |  |
| --- | --- | --- | --- |

| 7. | SCREENSHOTS OF EACH TASK OUTPUT |  |  |
| --- | --- | --- | --- |

TASK 1:

Design a delay subroutine to generate a waveform on PORTC.0 with 50% DC and 0.5

sec period.

TASK 2:

Implement a 8-bit counter to count on every rising edge of the above waveform. The

state of the counter needs to be displayed (display 8 bits only) on a 10-bit LED bar

connected to PORTB. Do not worry about the counter overflow.

TASK3:

Also connect the 9th and 10th bit of the LED bar to PORTC.5 and PORTC.6 pins.

Toggle PORTC.5 and PORTC.6 for every 5th rising pulses and every 10th rising

pulse of the counter respectively.

TASK4:

Modify the above code to use interrupts to update the status/toggle of the 9th and

10th LED.

See video for Tasks 2-4.

| 8. | SCREENSHOT OF EACH DEMO |  |  |
| --- | --- | --- | --- |

TASK 1:  Assembly without interrupt

TASK2:  C without interrupt

TASK3:  Assembly with interrupt

TASK4:  C with interrupt

| 9. | VIDEO LINKS OF EACH DEMO |  |  |
| --- | --- | --- | --- |
| https://youtu.be/42GUsY9HwiE |  |  |  |
| 10. | GOOGLECODE LINK OF THE DA |  |  |
| https://github.com/Wellsa15/wellsa_unlv |  |  |  |

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Andrew Wells